V

I

R

Name: _________________

Date:   _________________

Ohm’s Law 2

- (2 pts) R = 670 Ω and I = 0.3 A.  What is the voltage?

- (2 pts) V = 2.5 V and R = 1.0 kΩ.  What is the current?  Hint: metric prefixes (like k) are on p. 18 in the book.

- (2 pts) What is the current when R = 0.52 Ω and V = 6.7 V?

- (2 pts) What is the resistance when I = 0.158 A and V = 112 V?

- When R = 3.0 MΩ and I = 1.88 A, what is the voltage?

| Circuit 1 | Circuit 2 A: 170 Ω | Circuit 3 | Circuit 4 |
| --- | --- | --- | --- |
| 22 Ω 1.5 V | 1.5 V B: 15 Ω | 6.0 Ω | 4.1 Ω 4.0 Ω B A C 6.0 Ω 9.3 V |

- The following question refers to Circuit 1.  What current is going through the light bulb?

- The following questions refer to Circuit 2.

  - What is the total voltage across both light bulbs?

  - What is the equivalent resistance to light bulb A and light bulb B?

  - What is the current going through light bulb B?

- The following questions refer to Circuit 3.

- There is 2.0 A of current flowing through each light bulb.  What is the voltage of the battery?

- What is the resistance of the unlabeled light bulb?

- The following questions refer to Circuit 4.

- Are light bulbs A and B in series or in parallel?

- What is the total voltage across both light bulb A and B?

- What is the total resistance of both light bulb A and B?

- What is the current through light bulb A?

- Are light bulbs C and B in series or in parallel?

- What is the total voltage across both light bulb C and B?

- What is the equivalent resistance of both light bulb C and B?

- How much current is going through light bulb C?

- How much current is going through light bulb B?